- Copy the compressed raspbian operating system image to your laptop

- Uncompess the image

- Write the image to the micro SD drive

- After the write is complete you should have a drive named boot, you need to create an empty file on this “boot” drive named ssh.  NOTE: The file cannot have a file type extension!

  - Use windows file explore to list items in the “boot” drive on the micro ssd

  - On the View tab make sure “File Name Extentions” is checked

  - Off the home tab select “New Item→Text Document” and name the file ssh

  - You should get a pop up that tells you if you change the file extension the file might become unusable, click “Yes”.  NOTE: If you don’t get this pop up you must have created the file with a file extension, which means it will not work!!!!!

- From the tool bar Eject the micro sd card and put it into your raspberry pi.   NOTE: The sd card will be two drives, ejecting either one will work.

- Insert the micro SD into the raspberry pi

- We want to allow the raspberry pi to access the internet through your laptop so on the laptop

  - In search bar enter “network status” and select the network status application

  - Select “Change adapter options”, which is in white part of the screen

  - Right click on the icon for the wifi adaptor and select properties

  - Click on the sharing tab and:

    - Make sure “Allow other network users to connect through this computer” is checked

    - In the pull down for “Home networking connection” select cat 5 network adapter for your laptop.

- Connect your cat 5 cable between your laptop and the raspberry pi

- Connect raspberry pi to the provided micro USB power supply and the pi should boot.

- The red and green lights on the Pi board on the edge opposite the USB ports should light.  After a minute or so the green light will go off and mostly stay off, at which point you can advance

- Bring up a command window on the laptop and enter command:  arp -a

  - You’re looking for the section that has Interface 192.168.137.1 – It’ll look something like this:

Interface: 192.168.137.1 --- 0x1c

Internet Address      Physical Address      Type

192.168.137.140       b8-27-eb-3e-36-7c     static

192.168.137.255       ff-ff-ff-ff-ff-ff     static

224.0.0.2             01-00-5e-00-00-02     static

224.0.0.22            01-00-5e-00-00-16     static

224.0.0.251           01-00-5e-00-00-fb     static

224.0.0.252           01-00-5e-00-00-fc     static

239.255.255.250       01-00-5e-7f-ff-fa     static

255.255.255.255       ff-ff-ff-ff-ff-ff     static

  - Hopefully you’ll have a 192.168.137. XXX address that doesn’t end with 255, this should be the IP address for your pi, so remember this address

- Use putty and make an ssh connection to your pi’s IP address:

  - user name is pi and password is raspberry

- Start the vnc server on the pi by running: vncserver

- Check that the pi has a service listening on port 5901 via:  netstat -lnt

- On the laptop run the vnc client

  - Connect to the pi using the pi’s IP:5901